 RADIATION INCIDENT 

## Never Impede or Delay Medical Assistance

# Minor Spills:

Incidents which involve the release or spillage of less than 100 microcuries (Ci) of the radionuclide can generally be considered as “minor” spills.  In such cases:

- Notify all other persons in the room at once.

- Clear room of all persons except those needed to deal with the spill.

- Confine liquid spills by dropping absorbent media on the spill area.

- Confine solid spills by dampening and covering the spill area carefully and thoroughly.  Do not spread the contamination.

  - Use water, unless chemical reaction would release airborne contaminants.

  - Use oil otherwise.

- Notify the laboratory supervisor/principal investigator in-charge.

- Notify the Radiation Safety Office at 578-2008/578-2747.

- After normal business hours, notify the Campus Police 578-3231.

Major Spills or Release:

Incidents which occur outside a fume hood and involve the release or spillage of more than 100 microcuries (Ci) of the radionuclide should be considered “major” spills.  In such cases:

- In the event of suspected personal contamination, refer to Personal Contamination section below.

- Evacuate the spill area immediately.  Keep all persons who were present during the spill at a nearby, low-traffic, safe location.

- Contain the spill to minimize the spread of the contaminant.

- Secure the area with conspicuous notice.

- Notify the laboratory supervisor/principal investigator in-charge.

- Notify the Radiation Safety Office at 578-2008/578-2747.

- After normal business hours, notify Campus Police at 578-3231.

- Wait for assistance.

Personal Contamination:

- Notify Radiation Safety Office immediately at 578-2008/578-2747.  After normal business hours, notify Campus Police at 578-3231.

- Remove all contaminated personal objects (lab coat, gloves, clothing, shoes, etc.) and place them into a labeled radioactive waste bag.

- Measure and record the amount of contamination on the contaminated part of your body with a survey meter, if applicable.

- Wash the contaminated area with mild soap and water.

- Repeat washing as necessary.

- Never Impede or Delay Medical Assistance.